Camera Settings page

Check value (fwcamctlget.asp)

http://(IP ADDRESS)/asp-get/fwcamctlget.asp?FwModid=0

Example http://192.168.216.11/asp-get/fwcamctlget.asp?FwModid=0

Set value (fwcamctlset.cgi)

http://(IP ADDRESS)/cgi-bin/admin/fwcamctlset.cgi?[CGI(Set value)]=[IndexValue]&FwCgiVer=0x0001

↳What you want to set           ↳Essential

Example http://192.168.216.11/cgi-bin/admin/fwcamctlset.cgi? Brightness=128&FwCgiVer=0x0001 http://192.168.216.11/cgi-bin/admin/fwcamctlset.cgi? Tdn=2&Brightness=255&FwCgiVer=0x0001

|  | ASP(Get value) | CGI(Set value) | Index Value |
| --- | --- | --- | --- |
| Lens Type | LENS | Lens | value=0>ELC value=1>ALC |
| Brightness | BRIGHTNESS | Brightness | Default : 128, Range : [0, 255] |
| Shutter | SHUTTER | Shutter | value=0> Auto value=1> Manual value=2> Flicker |
| Lens ELC Mode | SHTMODE | ShtMode | value=0> Normal value=1> Deblur |
| Lens ALC Mode | DCMODE | DcMode | value=0> Indoor value=1> Outdoor value=2> Deblur |
| Manual Shutter Speed | SHUTTERSPEED | ShtterSpeed | value=0>1/30 value=1> 1/60 value=2> 1/120 value=3> 1/250 value=4> 1/500 value=5> 1/1000 value=6> 1/2000 value=7> 1/4000 value=8> 1/8000 value=9> 1/15000 value=10> 1/30000 |
| Sens Up | DSS | Dss | value=0>Off value=1> x2 value=2> x4 value=3> x8 value=4> x16 value=5> x32 |
| Agc | AGC | Agc | Default : 91, Range : [0, 255] |
| BackLight Mode | BACKLIGHT | BackLight | value=0>Off value=1> Hlc value=2> Blc value=3> Wdr |
| Hlc Level | HLMASKTHRS | HlMaskThrs | Default : 18, Range : [0, 20] |
| Hlc Mask Color | HLMASKCOLOR | HlMaskColor | value=0> White value=1> Yellow value=2> Cyan value=3> Green value=4> Magnet value=5> Red value=6> Blue value=7> Black |
| Blc H-Pos | BLCPOSX | BlcPosX | Default : 8, Range : [0, 20] |
| Blc V-Pos | BLCPOSY | BlcPosY | Default : 7, Range : [0, 20] |
| Blc H-Size | BLCSIZX | BlcSizX | Default : 3, Range : [0, 20] |
| Blc V-Size | BLCSIZY | BlcSizY | Default : 3, Range : [0, 20] |
| WDR Weight | WDRWEIGHT | WdrWeight | value=0> Low value=1> Middle value=2> High |
| Day & Night Mode | TDN | Tdn | value=0> Auto value=1> Color value=2> B&W value=3> Extern |
| Led On | LEDON | LedOn | value=0> off value=1> on |
| Anti Saturation | LEDSATCTRL | LedSatCtrl | Default : 0, Range : [0, 20] |
| AGC Threshold | TDNTHRS | TdnThrs | Default : 10, Range : [0, 20] |
| AGC Margin | TDNMARG | TdnMarg | Default : 10, Range : [0, 20] |
| Delay | TDNDLY | TdnDly | value=0> 1sec value=1> 3sec value=2> 5sec value=3> 10sec value=4> 15sec |
| IR Signal(Night) | TDNSW | TdnSw | value=0> High value=1> Low |
| Day to Night Threshold | TDNDNTHRES | TdnDnThres | Default : 9, Range : [0, 20] |
| Night to Day Threshold | TDNNDTHRES | TdnNdThres | Default : 7, Range : [0, 20] |
| White Balance Mode | AWB | Awb | value=0> Auto value=2> Preset value=3> Manual |
| Kelvin | AWBMNL | AwbMnl | value=0> 3000K value=1> 5000K value=2> 8000K |
| Manual RGain | RGAIN | Rgain | Default : 10, Range : [0, 20] |
| Manual BGain | BGAIN | Bgain | Default : 10, Range : [0, 20] |
| Color Gain | SATURATION | Saturation | Default :128, Range : [0, 255] |
| 2DNR Level | ADNR | Adnr | Default :8, Range : [0, 255] |
| 3DNR Level | ADNR3D | Adnr3D | Default :51, Range : [0, 255] |
| Sharpness | SHARPNESS | Sharpness | Default :128, Range : [0, 255] |
| Gamma | GAMMA | Gamma | value=0> 0.45 value=1> 0.55 value=2> 0.65 value=3> 0.75 |
| Mirror | MIRROR | Mirror | value=0> Off value=1> On |
| Flip | FLIP | Flip | value=0> Off value=1> On |
| D-Zoom | DZOOM | Dzoom | Default :10, Range : [1,64] |
| D-WDR | ACE | Ace | value=0> Off value=1> Low value=2> Middle value=3> High |
| Defog | DEFOG | Defog | value=0> Off value=1> On |
| Defog Mode | DEFOGMODE | DefogMode | value=0> Manual value=1> Auto |
| Defog Level | DEFOGLEVEL | DefogLevel | value=0> Low value=1> Middle value=2> High |

Web Admin – System – Date & Time – Time Zone List

Cf 1:  TimeZone - Australia/Sydney (All contents below should be entered.)

http://192.168.0.104/cgi-bin/admin/nsssetntpconf.cgi?

NssTimeZoneChange=1&

NssTimeZone=Australia/Sydney&

NssYear=2021&

NssMonth=12&

NssDay=14&

NssHour=8&

NssMinute=59&

NssSecond=10&

FwCgiVer=0x0001

http://192.168.0.48/cgi-bin/admin/nsssetntpconf.cgi?NssTimeZoneChange=1&NssTimeZone=Australia/Sydney&NssYear=2021&NssMonth=12&NssDay=14&NssHour=8&NssMinute=59&NssSecond=10&FwCgiVer=0x0001

CGI COMMAND WEB INPUT IMAGE

Cf 2:  TimeZone - Australia/Melbourne (All contents below should be entered.)

http://192.168.0.104/cgi-bin/admin/nsssetntpconf.cgi?

NssTimeZoneChange=1&

NssTimeZone=Australia/Melbourne&

NssYear=2021&

NssMonth=12&

NssDay=14&

NssHour=8&

NssMinute=59&

NssSecond=10&

FwCgiVer=0x0001

http://192.168.0.48/cgi-bin/admin/nsssetntpconf.cgi?NssTimeZoneChange=1&NssTimeZone=Australia/Melbourne&NssYear=2021&NssMonth=12&NssDay=14&NssHour=8&NssMinute=59&NssSecond=10&FwCgiVer=0x0001

CGI COMMAND WEB INPUT IMAGE

NTP time sync command

[IP address]/cgi-bin/admin/nsssyncntpserver.cgi?SvrAddr=’pool.ntp.org’&FwMkChg=YES&CgiVer=0x0001

http://192.168.0.53/cgi-bin/admin/nsssyncntpserver.cgi?SvrAddr='pool.ntp.org'&FwMkChg=YES&CgiVer=0x0001

Network Setting for Static or DHCP

STATIC : [IP Address]/ cgi-bin/admin/net_setnetworkv2.cgi?Eth0Ip=192.168.216.251&Eth0NetMask=255.255.0.0&Gateway= 192.168.0.1&Dns1=203.248.252.2&Dns2=164.124.101.2&Ipv6Enable=no&Eth0LinkLocal=fe80%3A%3A230%3A6fff%3Afe10%3A2b23%2F64&LanEthMode=static&SaveCfg=YES&FwRmChg=YES&CgiVer=0x0001&ChkNetChg=YES DHCP : [IP Address]/ cgi-bin/admin/net_setnetworkv2.cgi?Dns1=203.248.252.2&Dns2=164.124.101.2&LanEthMode=dhcp&SaveCfg= YES&FwRmChg=YES&CgiVer=0x0001&ChkNetChg=YES