Purpose of CGI Command (Model)

Update : 20180423

- Provide the command in CGI format of HTTP in order to make the setting and the searching easy and quick.

- CGI is 2 types. One is the setting and another is the searching.

Setting format: http://ipaddress/basic.cgi?<param>=<value>

Searching format: http:/ipaddress/basic.cgi?root.action.list=<value>

1. Video

- CGI Command : http://ipaddress/basic.cgi?root.video.<field>=<value>

- LIST Command : http://ipaddress/basic.cgi?root.action.list=video

- cf: http://192.168.1.201?basic.cgi?root.video.h264res1=1920x1080

Video can be simultaneously playback in main_h264, sub_h264

| Filed | Value | Description |
| --- | --- | --- |
| Main Video H264 |  |  |
| h264profile1 | baseline, main |  |
| h264res1 | 1920x1080, 1280x720, 800x600, 704x480, 704x400, 640x480 | Setting the resolution of h264. |
| h264fps1 | 30, 15, 10, 6, 5, 3, 2, 1 | Setting the framerate |
| h264gop1 | 1~150 | Setting GOP |
| h264quality1 | 0~51 | Setting Quality |
| h264bps1 | 128k, 256k, 512k, 1m ~ 12m | Setting bps |
| h264bitratemode1 | off, vbr, cbr | Setting bps mode |
| Sub Video H264 |  |  |
| h264res2 | 640x480, 640x360, 320x240 | Setting the resolution of h264. |
| h264fps2 | 30, 15, 10, 6, 5, 3, 2, 1 | Setting the framerate |
| h264gop2 | 1 ~ 150 | Setting GOP |
| h264quality2 | 0 ~ 50 | Setting Quality |
| h264bps2 | 128k,256k, 512k, 1~3m | Setting bps |
| h264bitratemode2 | off, vbr, cbr | Setting bps mode |
| Sub Video JPEG |  |  |
| jpegres2 | 1920x1080 | Setting the resolution of Jpeg |
| Jpegfps2 | 30, 15, 10, 6, 5, 3, 2, 1 | Setting the framerate |
| jpegquality2 | 1 ~ 159 | Setting Quality |
| jpegbps2 | 1m~20m, 25m, 30m | Setting bps |
| jpegbitratemode2 | off, vbr, cbr | Setting bps mode |

2.  ISP

- CGI Command : http://ipaddress/basic.cgi?root.isp.<field>=<value>

- LIST Command : http://ipaddress/basic.cgi?root.action.list=isp

- cf) http://192.168.1.201/basic.cgi?root.isp.sys_osddata=this_is_osddata

| field | value | dscription |
| --- | --- | --- |
| OSD |  |  |
| sys_osddata | string ( max length: 20) | “Alphabet a ~ z” is only allowed. Not available for the special symbol such as the space. |
| sys_osdmode | off, rightup, leftdown | Setting OSD text location |
| sys_osddton | on, off | Setting OSD date/time ON/OFF |
| Ouptput |  |  |
| sys_outputfreq | 50, 60 | Setting the frequency |
| WDNR |  |  |
| sys_wdnrmode | off, wdr, dnr | Setting WDR, DNR |
| Sys_wdnrlevel | Low, mid,high | Setting Level |
| Defog |  |  |
| sys_defogon | on, off | Defog on/off |
| sys_defogmode | manual, auto | Setting Defog mode |
| sys_defoglevel | low, mid, high | Setting Defog Level |
| Image |  |  |
| image_sharpness | 0 ~ 10 | Setting Sharpness |
| image_saturation | 0 ~ 20 | Setting Saturation |
| image_gamma | 0.45, 0.55, 0.66, 0.75 | Setting Gamma |
| image_mirroron | on, off | Mirror on/off |
| image_flipon | on, off | Flip on/off |
| image_ace | off, low, mid, high | Setting ACE |
| Shading |  |  |
| image_shadingon | on, off | Shading on/off |
| image_shadingweight | 0 ~ 200 | Setting Shading weight |
| Color |  |  |
| color_awb | auto, autoext, manual | Setting AWD mode |
| color_ctemp | 3000, 5000, 8000 | Setting CTemp value on AWE manual mode. |
| color_rgain | 0 ~ 20 | Setting Rgain value on AWE manual mode |
| color_bgain | 0 ~ 20 | Setting Bgain value on AWE manual mode |
| BackLight |  |  |
| backlight_mode | off, hlc, blc | Setting BackLight mode |
| backlight_hlclevel | 0 ~ 20 | Setting Level on BackLight HLC mode |
| backlight_hlccolor | white, yellow, cyon, green, magenta, red, blue, black, darkgreen | Setting Color on BackLight HLC mode |
| Backlight_blcblock | 0(xpos, ypos, xsize, ysize) xpos + xsize : 0 ~ 20 ypos + ysize : 0 ~ 20 | Setting BLC Block ( Only one cab be set) |
| DayNight |  |  |
| daynight_mode | auto, color, blkwht, extern | Setting DayNight mode |
| daynight_ispthres | 0 ~ 20 | Setting AGC Threshold on DayNight Auto mode |
| daynight_ispmargin | 0 ~ 20 | Setting AGC Margin on DayNight Auto mode |
| daynight_extthres | 0 ~ 20 | Setting D/N Threadhold on DayNight Extern mode. |
| daynight_ extmargin | 0 ~ 20 | Setting N/D Threadhold on DayNight Extern mode |
| daynight_externsw | high, low | Setting extern sw on DayNight Extern mode |
| daynight_delay | Low, mid, high | Setting “Delay” on DayNight Auto mode |
| IR |  |  |
| daynight_irledon | on, off | IR ON/OFF on DayNight Auto mode |
| daynight_antisat | 0 ~ 20 | Setting antisat on DayNight Auto mode |
| Exposure |  |  |
| expo_brightness | 0 ~ 20 | Setting Brightness |
| expo_shutter | auto, manual, flicher | Setting Shutter mode |
| expo_stmode | normal, deblur | Setting Mode |
| expo_stspeed | 50, 100, 200, 400, 800, 1600, 3200, 6400, 12800, 25600 | Setting the shutter speed on Shutter Manual mode |
| expo_agc | 0 ~ 10 | Setting AGC on Shutter Manual mode |

3. Network

- CGI Command : http://ipaddress/basic.cgi?root.network.<field>=<value>

- LIST Command : http://ipaddress/basic.cgi?root.action.list=network

- cf) http://192.168.1.201/basic.cgi?root.network.ipaddress=192.168.1.202

| field | value | description |
| --- | --- | --- |
| ipaddress | xxx.xxx.xxx.xxx | Setting IP address |
| gateway | xxx.xxx.xxx.xxx | Setting Gateway |
| netmask | x xx.xxx.xxx.xxx | Setting Netmask |
| dns1 | x xx.xxx.xxx.xxx | Setting First dns |
| dns1 | x xx.xxx.xxx.xxx | Setting Second dns |
| rtspport | 1 1024 ~ 65353 | Rtspport |
| httpport | 1 1024 ~ 65353 | Httpport |
| upnpon | on, off | Upnp on/off |
| upnpnip | String Max character : 15 | Upnp device id |
| upnpname | s string Max character : 32 | Setting Upnp name |
| dhcpon | on/off | Setting the network address on DHCP mode |

4. Date/Time

- CGI Command : http://ipaddress/basic.cgi?root.datetime.<field>=<value>

- LIST Command : http://ipaddress/basic.cgi?root.action.list=datetime

- cf) http://192.168.1.201/basic.cgi?root.datetime.statictime=2017.4.24.3.21.24

| filed | value | Description |
| --- | --- | --- |
| sntpon | on, off | Bring the time from SNTP server |
| sntpserver | string Max characters : 0 ~ 32 | Setting SNTP server |
| statictime | yyyy.mm.dd.hh.mm.ss | Setting the time cf)2017.4.24.3.21.24 |
| timezone | 0 ~ 27 | Setting the time zone |
| dst | on,off | Setting DST ON/FF |

5. Account

- CGI Command : http://ipaddress/basic.cgi?root.account.<field>=<value>

- LIST Command : http://ipaddress/basic.cgi?root.action.list=account

When transferring the encrypted password, it will be “root.account.user=”.

If it is not, it will be “root.account.nuser=”.

cf) The following shows the case that “1234” is encrypted to “TVRJek5BPT0=” and transferred

and another case that “1234” is transferred in “nuser” format as unencrypted.  Both two shows the same result.

- cf) http://192.168.1.201/basic.cgi?root.account.user=a0(admin,TVRJek5BPT0=,0)

- cf) http://192.168.1.201/basic.cgi?root.account.nuser=a0(admin,1234,0)

| field | value | Description |
| --- | --- | --- |
| user/nuser | d[index](id, 0,0) [index] : 0 ~ 4 | Delete USER. Cf) Delete id=”admin” on index #0. - d0(admin,0,0) |
| user/nuser | a[index](id, HashPw,level) [index] : 0 ~ 4 Level: 0~2 0: administrator 1: operator 2: guest - Refer to appendix for HashPW | Change User Information. cf) id=”admin”, pw=”1234” on Index 0, Add level=”administrator” - User format a0(admin,TVRJek5BPT0=,0) - Nuser format a0(admin,1234,0) |
| user/nuser | e[index](id, id, pw, level) [index] : 0 ~ 4 Level: 0~2 0: administrator 1: operator 2: guest | Add user. cf 1) Change the password Change the password “id=”admin”” on Index 0 to “3456”. - User format e0(admin,admin,TXpRMU5nPT0=,0) 2) nuser format e0(admin,admin,3456,0) Change the ID and the password. Change id=admin”, pw=”3456” to id=”root”, pw=”7890”. - user format e0(admin,root,TnpnNU1BPT0=,0) - nuser format e0(admin,root,7890,0) |

6. Motion

- CGI Command : http://ipaddress/Basic.cgi?root.motion.<field>=<value>

- LIST Command : http://ipdaddress/Basic.cgi?root.action.list=motion

- cf) http://192.168.1.201/basic.cgi?root.motion.block=1(3,3,10, 5)

| field | value | Description |
| --- | --- | --- |
| block | [num](xpos, ypos, xsize, ysize) num : 0 ~ 3 xpos + xsize : 0 ~ 60 ypos + ysize : 0 ~ 34 | Setting the motion detection area. cf) Setting #1 |
| block | [num](0,0,0,0) [num] : 0 ~ 3 | Disable the motion detection area. cf) Disable #1 window - 1(0,0,0,0) |
| alarmscreenon | on, off | Display the alarm when the motion is detected. |
| sensitivity | 0 ~ 10 | Setting the motion sensitivity |

7. Privacy

- CGI Command : http://ipaddress/basic.cgi?root.privacy.<field>=<value>

- LIST Command : http://ipaddress/basic.cgi?root.action.list=privacy

- cf) http://192.168.1.201/basic.cgi?root.privacy.block=1(3,3,10,5)

| field | value | Description |
| --- | --- | --- |
| block | [num](xpos, ypos, xsize, ysize) [num] : 0 ~ 7 xpos + xsize : 0 ~ 60 ypos + ysize : 0 ~ 34 | Setting Privacy Zone cf) Setting #1 window. - 1(3,3, 10,5) |
| block | [num](0,0,0,0) [num] : 0 ~ 7 | cf) Setting #1 window - 1(0,0,0,0) |
| ylevel | 0 ~ 20 | Setting the privacy window Color |
| cblevel | 0 ~ 20 |  |
| crLevel | 0 ~ 20 |  |
| translevel | 0, 1, 2, 3 | Setting the privacy window transparency |

8. OSD

- CGI Command : http://ipaddress/basic.cgi?root.osd.<field>=<value>

- LIST Command : http://ipaddress/basic.cgi?root.action.list=osd

| field | value | Description |
| --- | --- | --- |
| titletext | string ( max length: 20) | “Alphabet a ~ z“ is only allowed. The special mark such ‘space” is not allowed. |
| title | on, off | Title show |
| titlepos | rightup, leftdown | Setting Title location |
| titlecolor | green, white, gray, yellow | Setting Title font color |
| datetime | on, off | Date time show |
| datetimeampm | on, off | Setting AM/PM |
| datetimeline | one, two | Setting Line |
| datetimeweek | on, off | Display Week |
| datetimeformat | ymd, mdy | Setting Time format |
| datetimecolor | green, white, gray, yellow | Setting Date font color |

9. commoninfo

- CGI Command : http://ipaddress/basic.cgi?root.info.<field>=<value>

- LIST Command : http://ipaddress/basic.cgi?root.action.list=info

| field | value | Description |
| --- | --- | --- |
| productname | string ( max length: 24) | Allow the space between letter |
| devicename | string (max Length:24) | Allow the space between letter |
| manufacture | string ( max length: 24) | Allow the space between letter |
| contact | string ( max length: 24) | Allow the space between letter |
| fax | string ( max length: 24) | Allow the space between letter |
| email | string ( max length: 24) | Allow the space between letter |

EX) http://192.168.1.111/basic.cgi?root.info. productname =Front Camera

10. Audio

- CGI Command : http://ipaddress/basic.cgi?root.audio.<field>=<value>

| field | value | Description |
| --- | --- | --- |
| enable | On/off | Audio on/off |
| outgain | 0 ~ 127 | Audio volume level |

cf:) http://192.168.0.71/basic.cgi?root.audio.enable=off

11. RTSP
   - CGI Command : http://ipaddress/basic.cgi?root.rtsp.<field>=<value>

| field | value | Description |
| --- | --- | --- |
| auth | enable/disable | Check rtsp id/password |

12 . Custom Command

- CGI Command : http://ipaddress/basic.cgi? root.rtsp.ledmode=<value>

| command | value | Description |
| --- | --- | --- |
| root.rtsp.ledmode | 0, 1, 2 | 0: off 1: on 2: blink |

cf:)  http://192.168.1.201/basic.cgi? root.rtsp.ledmode=2    : Front led blink

http://192.168.1.201/basic.cgi? root.rtsp.ledmode=1    : Front led on

APPENDIX

Encryption code for the password when adding or changing the user account.

//============   Base64.h =============================//

#include <string.h>

#include <stdlib.h>

#pragma once

/*------ Base64 Encoding Table ------*/

static const char MimeBase64[] = {

'A', 'B', 'C', 'D', 'E', 'F', 'G', 'H',

'I', 'J', 'K', 'L', 'M', 'N', 'O', 'P',

'Q', 'R', 'S', 'T', 'U', 'V', 'W', 'X',

'Y', 'Z', 'a', 'b', 'c', 'd', 'e', 'f',

'g', 'h', 'i', 'j', 'k', 'l', 'm', 'n',

'o', 'p', 'q', 'r', 's', 't', 'u', 'v',

'w', 'x', 'y', 'z', '0', '1', '2', '3',

'4', '5', '6', '7', '8', '9', '+', '/',

};

/*------ Base64 Decoding Table ------*/

static int DecodeMimeBase64[256] = {

-1,-1,-1,-1,-1,-1,-1,-1,-1,-1,-1,-1,-1,-1,-1,-1,  /* 00-0F */

-1,-1,-1,-1,-1,-1,-1,-1,-1,-1,-1,-1,-1,-1,-1,-1,  /* 10-1F */

-1,-1,-1,-1,-1,-1,-1,-1,-1,-1,-1,62,-1,-1,-1,63,  /* 20-2F */

52,53,54,55,56,57,58,59,60,61,-1,-1,-1,-1,-1,-1,  /* 30-3F */

-1, 0, 1, 2, 3, 4, 5, 6, 7, 8, 9,10,11,12,13,14,  /* 40-4F */

15,16,17,18,19,20,21,22,23,24,25,-1,-1,-1,-1,-1,  /* 50-5F */

-1,26,27,28,29,30,31,32,33,34,35,36,37,38,39,40,  /* 60-6F */

41,42,43,44,45,46,47,48,49,50,51,-1,-1,-1,-1,-1,  /* 70-7F */

-1,-1,-1,-1,-1,-1,-1,-1,-1,-1,-1,-1,-1,-1,-1,-1,  /* 80-8F */

-1,-1,-1,-1,-1,-1,-1,-1,-1,-1,-1,-1,-1,-1,-1,-1,  /* 90-9F */

-1,-1,-1,-1,-1,-1,-1,-1,-1,-1,-1,-1,-1,-1,-1,-1,  /* A0-AF */

-1,-1,-1,-1,-1,-1,-1,-1,-1,-1,-1,-1,-1,-1,-1,-1,  /* B0-BF */

-1,-1,-1,-1,-1,-1,-1,-1,-1,-1,-1,-1,-1,-1,-1,-1,  /* C0-CF */

-1,-1,-1,-1,-1,-1,-1,-1,-1,-1,-1,-1,-1,-1,-1,-1,  /* D0-DF */

-1,-1,-1,-1,-1,-1,-1,-1,-1,-1,-1,-1,-1,-1,-1,-1,  /* E0-EF */

-1,-1,-1,-1,-1,-1,-1,-1,-1,-1,-1,-1,-1,-1,-1,-1   /* F0-FF */

};

const unsigned int MAX_BUF_SIZE = 1024;

class CBase64

{

public:

CBase64(void);

~CBase64(void);

private:

char* m_pszDecordText;

char* m_pszEncordText;

int base64_decode(char *text, unsigned char *dst, int numBytes );

int base64_encode(char *text, int numBytes, char **encodedText);

public:

BOOL Decord_Base64( CString strText, CString& strResult );

BOOL Encord_BASE64( CString strText, CString& strResult );

};

//==================== Base64.cpp ==============================//

#include "stdafx.h"

#include "Base64.h"

CBase64::CBase64(void)

{

// Init Original Data

m_pszDecordText = NULL;

m_pszEncordText = NULL;

}

CBase64::~CBase64(void)

{

// Delete Dynamic Data

if(m_pszDecordText){

delete [] m_pszDecordText;

m_pszDecordText = NULL;

}

if(m_pszEncordText){

delete [] m_pszEncordText;

m_pszEncordText = NULL;

}

}

int CBase64::base64_decode(char *text, unsigned char *dst, int numBytes )

{

const char* cp;

int space_idx = 0, phase;

int d, prev_d = 0;

unsigned char c;

space_idx = 0;

phase = 0;

for ( cp = text; *cp != '\0'; ++cp ) {

d = DecodeMimeBase64[(int) *cp];

if ( d != -1 ) {

switch ( phase ) {

case 0:

++phase;

break;

case 1:

c = ( ( prev_d << 2 ) | ( ( d & 0x30 ) >> 4 ) );

if ( space_idx < numBytes )

dst[space_idx++] = c;

++phase;

break;

case 2:

c = ( ( ( prev_d & 0xf ) << 4 ) | ( ( d & 0x3c ) >> 2 ) );

if ( space_idx < numBytes )

dst[space_idx++] = c;

++phase;

break;

case 3:

c = ( ( ( prev_d & 0x03 ) << 6 ) | d );

if ( space_idx < numBytes )

dst[space_idx++] = c;

phase = 0;

break;

}

prev_d = d;

}

}

return space_idx;

}

int CBase64::base64_encode(char *text, int numBytes, char **encodedText)

{

unsigned char input[3]  = {0,0,0};

unsigned char output[4] = {0,0,0,0};

int   index, i, j, size;

char *p, *plen;

plen           = text + numBytes - 1;

size           = (4 * (numBytes / 3)) + (numBytes % 3? 4 : 0) + 1;

(*encodedText) = (char*)malloc(size);

j              = 0;

for  (i = 0, p = text;p <= plen; i++, p++) {

index = i % 3;

input[index] = *p;

if (index == 2 || p == plen) {

output[0] = ((input[0] & 0xFC) >> 2);

output[1] = ((input[0] & 0x3) << 4) | ((input[1] & 0xF0) >> 4);

output[2] = ((input[1] & 0xF) << 2) | ((input[2] & 0xC0) >> 6);

output[3] = (input[2] & 0x3F);

(*encodedText)[j++] = MimeBase64[output[0]];

(*encodedText)[j++] = MimeBase64[output[1]];

(*encodedText)[j++] = index == 0? '=' : MimeBase64[output[2]];

(*encodedText)[j++] = index <  2? '=' : MimeBase64[output[3]];

input[0] = input[1] = input[2] = 0;

}

}

(*encodedText)[j] = '\0';

return size;

}

BOOL CBase64::Decord_Base64( CString strText, CString& strResult )

{

// Init String Buff

if(m_pszDecordText){

delete [] m_pszDecordText;

m_pszDecordText = NULL;

}

m_pszDecordText = new char[MAX_BUF_SIZE];

memset( m_pszDecordText, 0, sizeof(char)*MAX_BUF_SIZE );

// Change MBCS

CStringA strA = (CStringA)strText;

sprintf_s( m_pszDecordText, sizeof(char)*MAX_BUF_SIZE, strA.GetBuffer() );

char* pchBase64Decorded = NULL;

pchBase64Decorded = new char[MAX_BUF_SIZE];

memset( pchBase64Decorded, 0, sizeof(char)*MAX_BUF_SIZE );

int iBase64DecodeLen = base64_decode( m_pszDecordText, (unsigned char*)pchBase64Decorded, MAX_BUF_SIZE );

CStringA strResultA;

strResultA.Format("%s", pchBase64Decorded );

// Set Result

strResult = (CString)strResultA;

// Delete Dynamic Data

if(pchBase64Decorded){

delete [] pchBase64Decorded;

pchBase64Decorded = NULL;

}

if(m_pszDecordText){

delete [] m_pszDecordText;

m_pszDecordText = NULL;

}

return TRUE;

}

BOOL CBase64::Encord_BASE64( CString strText, CString& strResult )

{

// Init String Buffer

if(m_pszEncordText){

delete [] m_pszEncordText;

m_pszEncordText = NULL;

}

m_pszEncordText = new char[MAX_BUF_SIZE];

memset( m_pszEncordText, 0, sizeof(char)*MAX_BUF_SIZE );

// Change MBCS

CStringA strA = (CStringA)strText;

sprintf_s( m_pszEncordText, sizeof(char)*MAX_BUF_SIZE, strA.GetBuffer() );

// Encord BASE64

char* pchBase64Encorded = NULL;

int   iBase64EncodeLen  = base64_encode( m_pszEncordText, strlen(m_pszEncordText), &pchBase64Encorded );

CStringA strResultA;

strResultA.Format( "%s", pchBase64Encorded );

// Set Result

strResult = (CString)strResultA;

/*********************************************************

Caution !!

First Param is Plain Text Pointer

Second Param is Plain Text Length

Third Param is Result Double Pointer ( Just do Free !! )

*********************************************************/

if(pchBase64Encorded ){

free(pchBase64Encorded);

pchBase64Encorded = NULL;

}

if(m_pszEncordText){

delete [] m_pszEncordText;

m_pszEncordText = NULL;

}

return TRUE;

}

//====test.cpp ========//

MakeHashPw(CString _strSrc, CString &_strDst)

{

// Base 64 Encord

CString strResult = _T("");

CBase64 Base64;

// Make PW Hash

Base64.Encord_BASE64( _strSrc  , strResult );

Base64.Encord_BASE64( strResult, _strDst   );

return true;

}

Void main ()

{

CString strHashPW;

CString strCgiCommand;

//1234를 hashcode화 한다.

MakeHashPw(L“1234”, strHashPW);

//hashPw화된 값을 user형식에 대입한다.

strCgiCommand.Format (L”192.168.0.201/basic.cgi?root.account.user=a0(admin,%s,0)”, strHashPW);

sendCommand (strCgiCommand);

}